# TABLE DATA FROM: RETWALL BILL.pdf

This document contains properly formatted tables extracted from the PDF.

# Table 1 (Page 1)

| 1 | 1EarthworkinexcavationforfoundationofB-1 structure complete with all lift, dewatering, 1.1.4 shoring and shuttering etc. including refilling of trenches in 250 mm layer, ramming, watering and disposal of surplus soll within a lead of 1000 meters, as per drawing and technical Earth In excavation for foundation of specification. [MORTH Specification: Clause 304]. in ordinary soll [MORTH Specification clause 301] BY MECHANICAL MEANS Depth upto 3m | 935.74 | Cum | 57 | P Cum | 53337.32 |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Earthwork In excavation for foundation of B-1 structure complete with all lift, dewatering, 1.3.2 shoring and shuttering etc. including refilling of trenches in 250 mm layer, ramming, watering and disposal of surplus soil within a lead of 1000 meters, as per drawing and technical Earth in excavation for foundation of specification. [MORTH Specification Clause 304].In Ordinary/Soft rock (not requiring blasting) [MORTH Specification: Clause 3011 BY MECHANICAL MEANS | 382.44 | Cum | 83 | P Cum | 31742.51 |
| 3 | Structural cement concrete (Design Mix) in open foundations, with form work, at level. Cement concrete for plain/reinforced concrete in open foundations as per drawing and technical specification. [MORTH Specification: Section 1700 & 1500). With 20mm nominal size aggregate P.C.C. M-15 grade | 93.57 | Cum | 3038 | P Cum | 284278.57 |
| 4 | Structural cement concrete for plain concrete/ reinforced concrete for substructure at any level, complete as per [MORTH Specification: Section 1500 & 1700]. drawing and technical specification. Design mix concrete PCC M-15 Upto 5 mtr. Height | 1863.99 | cum | 3666 | P Cum | 6833377.26 |
| 6 | Providing A.C. pipe Weep holes 150mm B-3 dia in Masonry/plain Concrete/Reinforced 13 Concrete abutment, wing wall/return wall complete as per drawing and technical specification. [MORTH Specification : Clause 2205]. | 489.60 | R mtr | 114 | P RM | 55814.40 |

## Page 1 Text Content

RETAINING WALLS (PADLI GUJARESHWAR)
ABSTRACT OF COST
S.No. Particulars. Quantity Rate Unit Amount
1 1EarthworkinexcavationforfoundationofB-1 935.74 Cum 57 P Cum 53337.32
structure complete with all lift, dewatering, 1.1.4
shoring and shuttering etc. including
refilling of trenches in 250 mm layer, ramming,
watering and disposal of surplus soll within a
lead of 1000 meters, as per drawing and
technical Earth In excavation for foundation of
specification. [MORTH Specification: Clause
304]. in ordinary soll [MORTH Specification
clause 301] BY MECHANICAL MEANS Depth
upto 3m
2 Earthwork In excavation for foundation of B-1 382.44 Cum 83 P Cum 31742.51
structure complete with all lift, dewatering, 1.3.2
shoring and shuttering etc. including
refilling of trenches in 250 mm layer, ramming,
watering and disposal of surplus soil within a
lead of 1000 meters, as per drawing and
technical Earth in excavation for foundation of
specification. [MORTH Specification Clause
304].In Ordinary/Soft rock (not requiring
blasting) [MORTH Specification: Clause 3011
BY MECHANICAL MEANS
3 Structural cement concrete (Design Mix) in 93.57 Cum 3038 P Cum 284278.57
open foundations, with form work, at level.
Cement concrete for plain/reinforced concrete
in open foundations as per drawing and
technical specification. [MORTH Specification:
Section 1700 & 1500).
With 20mm nominal size aggregate
P.C.C. M-15 grade
4 Structural cement concrete for plain concrete/ 1863.99 cum 3666 P Cum 6833377.26
reinforced concrete for substructure at any
level, complete as per [MORTH Specification:
Section 1500 & 1700]. drawing and technical
specification. Design mix concrete PCC M-15
Upto 5 mtr. Height
6 Providing A.C. pipe Weep holes 150mm B-3 489.60 R mtr 114 P RM 55814.40
dia in Masonry/plain Concrete/Reinforced 13
Concrete abutment, wing wall/return wall
complete as per drawing and technical
specification. [MORTH Specification : Clause
2205].
Total Rs. 7258550.06

## Page 2 Text Content

RETAINING WALLS (PADLI GUJARESHWAR)
DETAILS OF MEASUREMENTS
S.N. Particulars Nos. Length Breadth Height Qty. Units
1 Earthwork in excavation for foundation of
structure complete with all lift, dewatering,
1.1.4 shoring and shuttering etc. including
refilling of trenches in 250 mm layer,
ramming, watering and disposal of surplus
soll within a lead of 1000 meters, as per
drawing and technical Earth In excavation
for foundation of specification. [MORTH
Specification: Clause 304]. in ordinary soll
[MORTH Specification clause 301] BY
MECHANICAL MEANS Depth upto 3m
Return Walls 1 1 x 15 x 2.22 x 1.50 = 49.88 Cum
1 x 15 x 2.77 x 1.50 = 62.25 Cum
1 x 15 x 2.73 x 1.50 = 61.50 Cum
1 x 15 x 2.63 x 1.50 = 59.25 Cum
1 x 15 x 2.68 x 1.50 = 60.38 Cum
1 x 15 x 2.67 x 1.50 = 60.00 Cum
1 x 15 x 2.60 x 1.50 = 58.50 Cum
1 x 15 x 2.52 x 1.50 = 56.63 Cum
1 x 15 x 2.65 x 1.50 = 59.63 Cum
1 x 15 x 2.68 x 1.50 = 60.38 Cum
1 x 2 x 2.68 x 1.50 = 8.05 Cum
Return Walls 2 1 x 15 x 2.42 x 1.50 = 54.38 Cum
1 x 15 x 2.33 x 1.50 = 52.50 Cum
1 x 7 x 2.12 x 1.50 = 22.23 Cum
1 x 15 x 2.28 x 1.50 = 51.38 Cum
1 x 15 x 2.27 x 1.50 = 51.00 Cum
Return Walls 3 1 x 6.95 x 2.27 x 1.50 = 23.63 Cum
Return Walls 4 1 x 6.9 x 2.68 x 1.50 = 27.77 Cum
Pipe Culvert 1 x 6.2 x 2.77 x 1.50 = 25.73 Cum
Pipe Culvert 1 x 7.4 x 2.77 x 1.50 = 30.71 Cum
Total Qty. 935.74 Cum
2 Earthwork Inexcavation for foundation of B-
1 structure complete with all lift, dewatering,
1.3.2 shoring and shuttering etc. including
refilling of trenches in 250 mm layer,
ramming, watering and disposal of surplus
soil within a lead of 1000 meters, as per
drawing and technical Earth in excavation
for foundation of specification. [MORTH
Specification Clause 304].In Ordinary/Soft
rock (not requiring blasting) [MORTH
Specification: Clause 3011 BY
MECHANICAL MEANS
Return Walls 1 1 x 15 x 2.22 x 0.45 = 14.96 Cum
1 x 15 x 2.77 x 0.50 = 20.75 Cum
1 x 15 x 2.73 x 1.25 = 51.25 Cum
1 x 15 x 2.63 x 1.90 = 75.05 Cum
1 x 15 x 2.68 x 0.90 = 36.23 Cum
1 x 15 x 2.67 x 0.45 = 18.00 Cum

## Page 3 Text Content

S.N. Particulars Nos. Length Breadth Height Qty. Units
1 x 15 x 2.60 x 0.60 = 23.40 Cum
1 x 15 x 2.52 x 0.75 = 28.31 Cum
1 x 15 x 2.65 x 0.25 = 9.94 Cum
1 x 15 x 2.68 x 0.35 = 14.09 Cum
1 x 2 x 2.68 x 0.35 = 1.88 Cum
Return Walls 2 1 x 15 x 2.42 x 0.30 = 10.88 Cum
1 x 15 x 2.33 x 0.35 = 12.25 Cum
1 x 7 x 2.12 x 0.35 = 5.19 Cum
1 x 15 x 2.28 x 0.35 = 11.99 Cum
1 x 15 x 2.27 x 0.35 = 11.90 Cum
Return Walls 3 1 x 6.95 x 2.27 x 0.45 = 7.09 Cum
Return Walls 4 1 x 6.9 x 2.68 x 0.50 = 9.26 Cum
Pipe Culvert 1 x 6.2 x 2.77 x 0.50 = 8.58 Cum
Pipe Culvert 1 x 7.4 x 2.77 x 0.56 = 11.47 Cum
Total Qty. 382.44 Cum
3 Structural cement concrete (Design Mix) in
open foundations, with form work, at level.
Cement concrete for plain/reinforced
concreteinopenfoundationsasperdrawing
and technical specification. [MORTH
Specification:
Section 1700 & 1500).
With 20mm nominal size aggregate
P.C.C. M-15 grade
Return Walls 1 1 x 15 x 2.22 x 0.15 = 4.99 Cum
1 x 15 x 2.77 x 0.15 = 6.23 Cum
1 x 15 x 2.73 x 0.15 = 6.15 Cum
1 x 15 x 2.63 x 0.15 = 5.93 Cum
1 x 15 x 2.68 x 0.15 = 6.04 Cum
1 x 15 x 2.67 x 0.15 = 6.00 Cum
1 x 15 x 2.60 x 0.15 = 5.85 Cum
1 x 15 x 2.52 x 0.15 = 5.66 Cum
1 x 15 x 2.65 x 0.15 = 5.96 Cum
1 x 15 x 2.68 x 0.15 = 6.04 Cum
1 x 2 x 2.68 x 0.15 = 0.81 Cum
Return Walls 2 1 x 15 x 2.42 x 0.15 = 5.44 Cum
1 x 15 x 2.33 x 0.15 = 5.25 Cum
1 x 7 x 2.12 x 0.15 = 2.22 Cum
1 x 15 x 2.28 x 0.15 = 5.14 Cum
1 x 15 x 2.27 x 0.15 = 5.10 Cum
Return Walls 3 1 x 6.95 x 2.27 x 0.15 = 2.36 Cum
Return Walls 4 1 x 6.9 x 2.68 x 0.15 = 2.78 Cum
Pipe Culvert 1 x 6.2 x 2.77 x 0.15 = 2.57 Cum
Pipe Culvert 1 x 7.4 x 2.77 x 0.15 = 3.07 Cum
Total Qty. 93.57 Cum

## Page 4 Text Content

S.N. Particulars Nos. Length Breadth Height Qty. Units
4 Structural cement concrete for plain
concrete/ reinforced concrete for
substructure at any level, complete as per
[MORTH Specification: Section 1500 &
1700]. drawing and technical specification.
Designmixconcrete PCCM-15Upto5mtr.
Height
Return Walls 1 1 x 15 x 1.92 x 2.9 = 83.38 Cum
1 x 15 x 2.47 x 4.1 = 151.70 Cum
1 x 15 x 2.43 x 4.3 = 156.95 Cum
1 x 15 x 2.33 x 4.3 = 150.50 Cum
1 x 15 x 2.38 x 4.3 = 153.73 Cum
1 x 15 x 2.37 x 4.25 = 150.88 Cum
1 x 15 x 2.30 x 4.35 = 150.08 Cum
1 x 15 x 2.22 x 4.1 = 136.33 Cum
1 x 15 x 2.35 x 4.2 = 148.05 Cum
1 x 15 x 2.38 x 4.3 = 153.73 Cum
1 x 2 x 2.38 x 4.3 = 20.50 Cum
Return Walls 2 1 x 15 x 2.12 x 2.90 = 92.08 Cum
1 x 15 x 2.03 x 2.95 = 89.98 Cum
1 x 7 x 1.82 x 2.60 = 33.06 Cum
1 x 15 x 1.98 x 2.95 = 87.76 Cum
1 x 15 x 1.97 x 2.90 = 85.55 Cum
Return Walls 3 1 x 6.95 x 4.6 x 0.15 = 4.80 Cum
Return Walls 4 1 x 6.9 x 4.85 x 0.15 = 5.02 Cum
Pipe Culvert 1 x 6.2 x 4.85 x 0.15 = 4.51 Cum
Pipe Culvert 1 x 7.4 x 4.9 x 0.15 = 5.44 Cum
Total Qty. 1863.99 Cum
5 Providing A.C. pipe Weep holes 150mm B-
3 dia in Masonry/plain Concrete/Reinforced
13 Concrete abutment, wing wall/return wall
complete as per drawing and technical
specification. [MORTH Specification :
Clause 2205].
Return Walls 1 1 x 102 x 1.10 = 112.20 RM
1 x 102 x 1.60 = 163.20 RM
Return Walls 2 1 x 46 x 1.10 = 50.60 RM
1 x 46 x 1.60 = 73.60 RM
Return Walls 3 1 x 6 x 1.10 = 6.60 RM
1 x 6 x 1.60 = 9.60 RM
Return Walls 4 1 x 6 x 1.10 = 6.60 RM
1 x 6 x 1.60 = 9.60 RM
Pipe Culvert 2 x 6 x 1.10 = 13.20 RM
2 x 6 x 1.60 = 19.20 RM
2 x 6 x 2.10 = 25.20 RM
Total Qty. 489.60 RM

## Page 5 Text Content

S.N. Particulars Nos. Length Breadth Height Qty. Units