# TABLE DATA FROM: r3.pdf

This document contains properly formatted tables extracted from the PDF.

# Table 1 (Page 1)

| SCHEDULE OF REINFORCEMENT FOR PIER CAP |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE OF BAR | SHAPE | LOCATION | DIREC TION | DIA OF BAR IN MM | SPACI NG IN MM | NO. OF BARS | LENGTH OF EACH BAR (M) | TOTAL LENG TH (M) | Unit Weig ht Kg/M | TOTAL WEIGHT KgS. |
| a |  | VER STIRRUPS IN LONG DIRECTION | LONGI TUDIN AL | 20 |  | 5 | 36.97 | 231.1 | 2.47 | 571 |
| s1 |  | STIRRUPS IN SHORT DIRECTION | STRIR RUPS | 12 | 300 | 61 | 5.35 | 407.9 | 0.89 | 364 |
| s2 |  | PROJECTE D PORTION | STRIR RUPS | 12 | 100 | 30 | 4.75 | 178.1 | 0.89 | 159 |
| h1 |  | HORIZONTA L STIRRUPS IN LONG DIRECTION | STRIR RUPS | 12 | 240 | 3 | 38.75 | 145.3 | 0.89 | 130 |
|  |  | TOTAL IN ONE PIER CAP |  |  |  |  |  |  |  | 1224 |
| SCHEDULE OF REINFORCEMENT FOR ABUTMENTR CAP |  |  |  |  |  |  |  |  |  |  |
| TYPE OF BAR | SHAPE | LOCATION | DIREC TION | DIA OF BAR IN MM | SPACI NG IN MM | NO. OF BARS | LENGTH OF EACH BAR (M) | TOTAL LENG TH (M) | Unit Weig ht Kg/M | TOTAL WEIGHT KgS. |
| a |  | BOTTOM | LONGI TUDIN AL | 16 |  | 8 | 18.40 | 184 | 1.58 | 291 |
| b |  | TOP | LONGI TUDIN AL | 16 |  | 8 | 18.40 | 184 | 1.58 | 291 |
| c |  |  | STRIR RUPS | 12 | 300 | 61 | 2.55 | 194.4 | 0.89 | 174 |
| d |  | SIDE FACE |  | 12 |  | 4 | 19.70 | 98.5 | 0.89 | 88 |
|  |  | TOTAL IN ONE ABUTMENT CAP |  |  |  |  |  |  |  | 844 |

## Page 1 Text Content

SCHEDULE OF REINFORCEMENT FOR PIER CAP
TYPE SHAPE LOCATION DIREC DIA SPACI NO. OF LENGTH OF TOTAL Unit TOTAL
OF TION OF NG IN BARS EACH BAR Weig WEIGHT
BAR BAR MM (M) LENG ht KgS.
IN MM TH (M) Kg/M
a VER LONGI 20 5 36.97 231.1 2.47 571
STIRRUPS TUDIN
IN LONG AL
DIRECTION
s1 STIRRUPS STRIR 12 300 61 5.35 407.9 0.89 364
IN SHORT RUPS
DIRECTION
s2 PROJECTE STRIR 12 100 30 4.75 178.1 0.89 159
D PORTION RUPS
h1 HORIZONTA STRIR 12 240 3 38.75 145.3 0.89 130
L RUPS
STIRRUPS
IN LONG
DIRECTION
TOTAL IN ONE PIER CAP 1224
SCHEDULE OF REINFORCEMENT FOR ABUTMENTR CAP
TYPE SHAPE LOCATION DIREC DIA SPACI NO. OF LENGTH OF TOTAL Unit TOTAL
OF TION OF NG IN BARS EACH BAR Weig WEIGHT
BAR BAR MM (M) LENG ht KgS.
IN MM TH (M) Kg/M
a BOTTOM LONGI 16 8 18.40 184 1.58 291
TUDIN
AL
b TOP LONGI 16 8 18.40 184 1.58 291
TUDIN
AL
c STRIR 12 300 61 2.55 194.4 0.89 174
RUPS
d SIDE FACE 12 4 19.70 98.5 0.89 88
TOTAL IN ONE ABUTMENT CAP 844